Command set for VPT (02.04.2025)

FW ver. 0.04

| Command from PC | Response from SPAN | Description |
| --- | --- | --- |
| VN | N:xx.xx | Get firmware version |
| ID | VPT | Get device identification name |
| HW | HW:xx | Get hardware revision |
| HELP | - | Get list of supported commands (help screen) |
| Readout commands |  |  |
| GCI | CI:xx.xx | Get instant current value, A |
| GCP | CP:xx.xx | Get peak current value, A. Will be reset after readout |
| GCS | CS:xx.xx | Get measured current value from servo, A |
| GPS | PS:xxx.x | Get position from servo, degrees |
| GS | S:xxxx | Get VPT status (see below) |
| GTS | TS:xx | Get measured temperature from servo, °C |
| GT | T:xx.x | Get module temperature, °C |
| GUM | UM:xx.x | Get measured voltage from servo, V |
| GUS | US:xx.x | Get power supply voltage, V |
| GU | U:xx.x | Get feedback/LineB voltage, V |
| GVS | VS:xxxxxxxxxxx | Get version string (RS485, CAN) |
| GWT | WT:xxxx:xx:xx | Get working time, hours:minutes:seconds |
| Setup commands |  |  |
| SBx (dec) | OK | Set baudrate for motor communication - PWM: not applicable - RS485: baudrate as integer - CAN: baudrate in kbits |
| SIx (dec) | OK | Set interface type |
| SIDx (dec) | OK | Set servo ID for bus protocols (RS485, CAN) |
| SCLx (dec) | OK | Set current limitation, A |
| Action commands (Do something) |  |  |
| DPxx.x (float) | OK | Move to position x, degrees |
| Read/Write EEPROM data |  |  |
| RBxx (hex) | B:yy | Read one byte from EEPROM at address x (HEX value) |
| WBxx:yy (hex:hex) | OK | Write one byte y to EEPROM at address x (HEX values) |
| Calibration commands |  |  |
| CC | OK | Calibrate current measurement (zero-point offset) |
| CUSxx.x (float) | OK | Calibrate power supply voltage measurement (nominal voltage) |
| Hardware commands |  |  |
| PWRx (dec) | OK | Set power to servo (0 – Off, 1 – On) |
| RAWxxxxxxxx | yyyyyyyy | Send raw command (RS485, CAN) |
| Debugging commands (for internal usage only!) |  |  |
| ADCx (dec) | xxx | Get raw ADC value for channel x as HEX |
| DACx (dec) | OK | Set raw value for DAC output (used in current limiter) |
| OUTx:y (dec:dec) | OK | Set output value of the pin (x – output number, y = 0 – off, 1 – on) |
| PSWx (dec) | OK | Enter password for hidden commands |
| Bootloader commands |  |  |
| BLS | OK | Start servo firmware bootloader |
| BLQ | OK | Quit servo bootloader |
| BLPx (dec) | OK | Set PCB ID for update (duplex only) |
| BLFxx + data | OK | Write one page to servo flash memory (BLFxx, xx - page number as HEX) |
| BLCx:y (hex:hex) | OK | Check Flash page (BLCxx:yyyy, xx - page number as HEX, yyyy - CRC16 as HEX) |
| BLExxxx:yy (hex:hex) | OK | Write byte value to EEPROM (BLExxxx:yy, xxxx - address as HEX, yy - value as HEX) |

Every command is translated to upper case and ends with \r sign. \ESC sign is used to reset receiving buffer and start new transmission. Every command should be answered with requested data or OK/Error code.

Error codes

E.C – wrong command

E.P – wrong command parameters

E.H – hardware problems

E.M – wrong working mode selected

E.S – wrong password

| Interface ID | Description |
| --- | --- |
| 0 | PWM |
| 1 | RS485 |
| 2 | RS485 for big actuators |
| 3 | RS485 for new actuators |
| 4 | Volz CAN |
| 5 | Volz CAN (old bootloader, the rest of the functionality is identical) |
| 6 | Amazon |
| 7 | SBus |
| 8 | UAVCAN_v0 |
| 9 | Meteor CAN |